CISCO PACKET TRACER Guided Lab Project Multi-Floor Office Network with Dual ISP Failover Beginner Level ¦ Estimated Time: 3–4 Hours

| Skills Covered VLANs & Trunking Inter-VLAN Routing OSPF Dynamic Routing Dual ISP Failover DHCP Server Setup | Devices Used 2x Cloud (ISP) 2x ISR4331 Routers 2x 3650 Core Switches 3x 2960 Access Switches PCs, Printers, APs | Learning Goals Build from scratch Understand each command Test & verify every step Simulate ISP failure Troubleshoot issues |
| --- | --- | --- |

# Project Overview

In this lab, you will build a realistic multi-floor office network from scratch in Cisco Packet Tracer. The network connects three floors of an office building to the internet through two separate ISPs. If one ISP goes down, the network automatically switches to the backup ISP — this is called ISP failover, and it is a real-world feature used in enterprise networks everywhere.

By the end of this project, you will understand:

- How routers and switches connect to form a hierarchical network

- What VLANs are and why offices use them to separate traffic

- How OSPF routing protocol works and why it is better than static routes

- How floating static routes create automatic ISP failover

- How to test your network and troubleshoot problems

## Network Topology Summary

The network has four logical layers stacked on top of each other:

| Layer | Devices | Purpose |
| --- | --- | --- |
| Internet / ISP | Cloud0 (ISP1), Cloud1 (ISP2) | Simulates your internet service providers |
| Edge / WAN | Router-R1, Router-R2 | Connects the office to both ISPs; handles failover |
| Core / Distribution | Core-SW1, Core-SW2 | High-speed backbone; routes between all VLANs |
| Access | ACC-SW1 (Floor 1), ACC-SW2 (Floor 2), ACC-SW3 (Floor 3) | Connects end devices on each floor |

## IP Address Plan

Use this table as your reference throughout the project. Every IP must be entered exactly as shown.

| Device | Interface | IP Address | Subnet Mask | Purpose |
| --- | --- | --- | --- | --- |
| ISP1 Cloud | Se0/3/0 | 203.0.113.1 | 255.255.255.252 | WAN link to R1 |
| Router-R1 | Se0/1/0 | 203.0.113.2 | 255.255.255.252 | Uplink to ISP1 |
| ISP2 Cloud | Se0/3/0 | 198.51.100.1 | 255.255.255.252 | WAN link to R2 |
| Router-R2 | Se0/1/0 | 198.51.100.2 | 255.255.255.252 | Uplink to ISP2 |
| R1 <-> Core-SW1 | Gi0/0 | 10.0.0.1 | 255.255.255.252 | LAN uplink |
| Core-SW1 | Gi1/0/1 (routed) | 10.0.0.2 | 255.255.255.252 | Uplink to R1 |
| R2 <-> Core-SW2 | Gi0/0 | 10.0.0.5 | 255.255.255.252 | LAN uplink |
| Core-SW2 | Gi1/0/1 (routed) | 10.0.0.6 | 255.255.255.252 | Uplink to R2 |
| Core-SW1 SVI | VLAN 10 | 192.168.10.1 | 255.255.255.0 | Floor 1 gateway |
| Core-SW1 SVI | VLAN 20 | 192.168.20.1 | 255.255.255.0 | Floor 2 gateway |
| Core-SW1 SVI | VLAN 30 | 192.168.30.1 | 255.255.255.0 | Floor 3 gateway |
| Floor 1 PCs | NIC | 192.168.10.0/24 | 255.255.255.0 | DHCP from Core-SW1 |
| Floor 2 PCs | NIC | 192.168.20.0/24 | 255.255.255.0 | DHCP from Core-SW1 |
| Floor 3 PCs | NIC | 192.168.30.0/24 | 255.255.255.0 | DHCP from Core-SW1 |

Phase 1: Build the Topology Add all devices and cable them together in Packet Tracer

## Adding Devices

Open Packet Tracer and start a new blank project. Add the following devices from the bottom panel:

| 1 | Add the ISP clouds |
| --- | --- |

From the bottom device panel, click Network Devices > WAN Emulation > Cloud-PT. Drag two clouds onto the canvas and name them ISP1-Rogers and ISP2-ATT by double-clicking the label under each device.

What is a Cloud device? In Packet Tracer, a Cloud device simulates an internet service provider or any external network you do not control. It gives your routers somewhere to connect to for internet traffic.

| 2 | Add the routers |
| --- | --- |

Click Network Devices > Routers > ISR4331. Drag two routers onto the canvas below the clouds. Name them Router-R1 (left) and Router-R2 (right).

Important: The ISR4331 only has Gigabit ports by default. You need to add a WAN serial module to connect to the ISP clouds. Double-click the router, go to the Physical tab, power it off, drag the HWIC-2T module into an empty slot, then power it back on.

| 3 | Add the core switches |
| --- | --- |

Click Network Devices > Switches > 3650-24PS. Drag two switches below the routers. Name them Core-SW1 (left) and Core-SW2 (right). These are Layer 3 switches, meaning they can route traffic between VLANs just like a router.

| 4 | Add the access switches |
| --- | --- |

Click Network Devices > Switches > 2960-24TT. Drag three switches at the bottom of the canvas, one for each floor. Name them ACC-SW1-Floor1, ACC-SW2-Floor2, and ACC-SW3-Floor3.

| 5 | Add end devices |
| --- | --- |

For each floor, add the following end devices from End Devices:

- Floor 1 (ACC-SW1): 2x PC, 1x Printer, 2x AccessPoint-PT (for wireless)

- Floor 2 (ACC-SW2): 1x PC, 1x Printer, 2x AccessPoint-PT

- Floor 3 (ACC-SW3): 1x PC, 1x Printer, 2x AccessPoint-PT

Tip: Place devices close to their floor switch. A neat layout saves a lot of confusion later when you are troubleshooting.

## Cabling the Devices

Now connect everything with cables. Use Copper Straight-Through for all connections unless noted otherwise.

| 6 | Connect ISPs to routers (WAN links) |
| --- | --- |

Use a Serial DCE cable for these connections. Go to Connections (lightning bolt icon) > Serial DCE.

- ISP1-Rogers Se0/3/0  →  Router-R1 Se0/1/0

- ISP2-ATT Se0/3/0  →  Router-R2 Se0/1/0

Serial vs Ethernet: The ISP connection uses a Serial cable because real WAN links are serial connections. The DCE end sets the clock speed, which is required for serial links to work. The cloud device acts as the DCE (clock source) end.

| 7 | Connect routers to core switches |
| --- | --- |

Use Copper Straight-Through cable:

- Router-R1 Gi0/0  →  Core-SW1 Gi1/0/1

- Router-R2 Gi0/0  →  Core-SW2 Gi1/0/1

| 8 | Connect core switches together (trunk link) |
| --- | --- |

This is the backbone link between the two core switches. Use Copper Straight-Through:

- Core-SW1 Gi1/0/24  →  Core-SW2 Gi1/0/24

Why connect the two core switches? This link lets traffic flow between them if one router or ISP goes down. It also allows VLAN traffic to spread across both switches.

| 9 | Connect core switches to access switches |
| --- | --- |

Use Copper Straight-Through cable:

- Core-SW1 Gi1/0/2  →  ACC-SW1-Floor1 Gi0/1  (trunk uplink)

- Core-SW1 Gi1/0/3  →  ACC-SW2-Floor2 Gi0/1  (trunk uplink)

- Core-SW2 Gi1/0/2  →  ACC-SW3-Floor3 Gi0/1  (trunk uplink)

| 10 | Connect end devices to access switches |
| --- | --- |

Use Copper Straight-Through. Connect each PC, printer, and access point to any available FastEthernet port on its floor switch. Keep note of which port each device is on — you will need to assign VLANs to those ports later.

Phase 2: Configure the Access Switches Set up VLANs and access ports on each floor switch

VLANs (Virtual Local Area Networks) logically separate your network into groups. Even though all three floors share the same physical switches and cables, VLANs keep their traffic isolated. Floor 1 cannot talk to Floor 2 unless the traffic goes through the core switch router, where you can apply security rules.

Key concept — VLANs: Think of a VLAN like a separate hallway inside a building. Floor 1 has its own hallway (VLAN 10), Floor 2 has its own (VLAN 20), and Floor 3 has its own (VLAN 30). Traffic in one hallway cannot enter another unless it goes through a controlled checkpoint (the Layer 3 core switch).

## Configure ACC-SW1 (Floor 1 — VLAN 10)

Click on ACC-SW1-Floor1 > CLI tab. Press Enter to get a prompt, then type the following commands exactly:

### Step A — Enter global configuration mode

Switch> enable Switch# configure terminal Switch(config)# hostname ACC-SW1

### Step B — Create VLAN 10 and name it

ACC-SW1(config)# vlan 10 ACC-SW1(config-vlan)# name Floor1-Users ACC-SW1(config-vlan)# exit

What just happened? You created VLAN 10 in the switch's database and gave it a human-readable name. The switch now knows VLAN 10 exists, but no ports are assigned to it yet.

### Step C — Assign access ports to VLAN 10

Replace Fa0/2, Fa0/3, Fa0/4 with the actual ports your PCs, printers, and APs are connected to:

ACC-SW1(config)# interface range Fa0/2 - 4 ACC-SW1(config-if-range)# switchport mode access ACC-SW1(config-if-range)# switchport access vlan 10 ACC-SW1(config-if-range)# spanning-tree portfast ACC-SW1(config-if-range)# exit

What does 'switchport mode access' do? It tells the switch this port connects to an end device (PC, printer) rather than another switch. Access ports carry traffic for only ONE VLAN. The 'spanning-tree portfast' command makes the port come up immediately instead of waiting 30 seconds.

### Step D — Configure the uplink to Core-SW1 as a trunk

The port connecting to Core-SW1 must be a trunk port because it carries traffic from ALL VLANs:

ACC-SW1(config)# interface Gi0/1 ACC-SW1(config-if)# switchport mode trunk ACC-SW1(config-if)# switchport trunk allowed vlan 10,20,30 ACC-SW1(config-if)# exit

Trunk vs Access ports: A trunk port is like a multi-lane highway — it carries traffic from multiple VLANs at once using VLAN tags (802.1Q). An access port is a single-lane road — it carries traffic from one VLAN only, with no tags.

### Step E — Save the configuration

ACC-SW1(config)# end ACC-SW1# write memory Building configuration... [OK]

Always save! If Packet Tracer crashes or you close it, unsaved configurations are lost. Use 'write memory' or 'copy running-config startup-config' after every major step.

## Configure ACC-SW2 (Floor 2 — VLAN 20) and ACC-SW3 (Floor 3 — VLAN 30)

Repeat the exact same steps for ACC-SW2 and ACC-SW3, with these differences:

| Switch | Hostname | VLAN Number | VLAN Name |
| --- | --- | --- | --- |
| ACC-SW1-Floor1 | ACC-SW1 | 10 | Floor1-Users |
| ACC-SW2-Floor2 | ACC-SW2 | 20 | Floor2-Users |
| ACC-SW3-Floor3 | ACC-SW3 | 30 | Floor3-Users |

For ACC-SW3, its uplink goes to Core-SW2 (Gi1/0/2) instead of Core-SW1. The trunk configuration is the same.

Phase 3: Configure the Core Switches Set up inter-VLAN routing and uplinks to the routers

The core switches are the brains of your internal network. Because they are Layer 3 switches, they can both switch frames at Layer 2 AND route packets at Layer 3 between VLANs. This means Floor 1 can talk to Floor 2 without the traffic ever leaving the building.

## Configure Core-SW1

Click on Core-SW1 > CLI tab and enter the following:

### Step A — Basic setup and enable IP routing

Switch> enable Switch# configure terminal Switch(config)# hostname Core-SW1 Switch(config)# ip routing

What does 'ip routing' do? 'ip routing' turns on Layer 3 routing capability on the switch. Without this command, the 3650 switch acts like a regular Layer 2 switch and cannot route between VLANs. This is the most important command for a Layer 3 switch.

### Step B — Create all three VLANs

Core-SW1(config)# vlan 10 Core-SW1(config-vlan)# name Floor1-Users Core-SW1(config-vlan)# exit Core-SW1(config)# vlan 20 Core-SW1(config-vlan)# name Floor2-Users Core-SW1(config-vlan)# exit Core-SW1(config)# vlan 30 Core-SW1(config-vlan)# name Floor3-Users Core-SW1(config-vlan)# exit

### Step C — Create SVI (Switch Virtual Interface) for each VLAN

An SVI is a virtual interface that acts as the default gateway for all devices in that VLAN. When a PC in VLAN 10 wants to talk to a PC in VLAN 20, it sends the packet to this gateway IP.

Core-SW1(config)# interface vlan 10 Core-SW1(config-if)# ip address 192.168.10.1 255.255.255.0 Core-SW1(config-if)# no shutdown Core-SW1(config-if)# exit Core-SW1(config)# interface vlan 20 Core-SW1(config-if)# ip address 192.168.20.1 255.255.255.0 Core-SW1(config-if)# no shutdown Core-SW1(config-if)# exit Core-SW1(config)# interface vlan 30 Core-SW1(config-if)# ip address 192.168.30.1 255.255.255.0 Core-SW1(config-if)# no shutdown Core-SW1(config-if)# exit

### Step D — Configure trunk port to ACC-SW1 and ACC-SW2

Core-SW1(config)# interface Gi1/0/2 Core-SW1(config-if)# switchport mode trunk Core-SW1(config-if)# switchport trunk allowed vlan 10,20,30 Core-SW1(config-if)# exit Core-SW1(config)# interface Gi1/0/3 Core-SW1(config-if)# switchport mode trunk Core-SW1(config-if)# switchport trunk allowed vlan 10,20,30 Core-SW1(config-if)# exit

### Step E — Configure trunk port to Core-SW2

Core-SW1(config)# interface Gi1/0/24 Core-SW1(config-if)# switchport mode trunk Core-SW1(config-if)# switchport trunk allowed vlan 10,20,30 Core-SW1(config-if)# exit

### Step F — Configure the uplink to Router-R1 as a routed port

This port connects to a router, not a switch, so it must be a Layer 3 routed port (no switchport). Give it an IP address from the point-to-point /30 subnet:

Core-SW1(config)# interface Gi1/0/1 Core-SW1(config-if)# no switchport Core-SW1(config-if)# ip address 10.0.0.2 255.255.255.252 Core-SW1(config-if)# no shutdown Core-SW1(config-if)# exit

Why /30 subnet? A /30 subnet has exactly 4 addresses: network, two usable hosts, and broadcast. It is perfect for point-to-point links between two devices where you never need more than two IPs. It wastes the fewest addresses.

### Step G — Configure DHCP pools for each floor

This turns Core-SW1 into a DHCP server, automatically assigning IP addresses to all PCs and devices on each floor:

Core-SW1(config)# ip dhcp pool FLOOR1 Core-SW1(dhcp-config)# network 192.168.10.0 255.255.255.0 Core-SW1(dhcp-config)# default-router 192.168.10.1 Core-SW1(dhcp-config)# dns-server 8.8.8.8 Core-SW1(dhcp-config)# exit Core-SW1(config)# ip dhcp pool FLOOR2 Core-SW1(dhcp-config)# network 192.168.20.0 255.255.255.0 Core-SW1(dhcp-config)# default-router 192.168.20.1 Core-SW1(dhcp-config)# dns-server 8.8.8.8 Core-SW1(dhcp-config)# exit Core-SW1(config)# ip dhcp pool FLOOR3 Core-SW1(dhcp-config)# network 192.168.30.0 255.255.255.0 Core-SW1(dhcp-config)# default-router 192.168.30.1 Core-SW1(dhcp-config)# dns-server 8.8.8.8 Core-SW1(dhcp-config)# exit Core-SW1(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.10 Core-SW1(config)# ip dhcp excluded-address 192.168.20.1 192.168.20.10 Core-SW1(config)# ip dhcp excluded-address 192.168.30.1 192.168.30.10

Excluded addresses: The excluded-address command reserves the first 10 IPs in each subnet for static assignment (gateways, servers, printers). DHCP will only hand out .11 and above to end devices.

Core-SW1(config)# end Core-SW1# write memory

## Configure Core-SW2

Core-SW2 is simpler — it is a redundant path, not the primary router. Configure it to carry VLAN traffic and connect to Router-R2:

Switch> enable Switch# configure terminal Switch(config)# hostname Core-SW2 Switch(config)# ip routing Core-SW2(config)# vlan 10 Core-SW2(config-vlan)# name Floor1-Users Core-SW2(config-vlan)# exit Core-SW2(config)# vlan 20 Core-SW2(config-vlan)# name Floor2-Users Core-SW2(config-vlan)# exit Core-SW2(config)# vlan 30 Core-SW2(config-vlan)# name Floor3-Users Core-SW2(config-vlan)# exit Core-SW2(config)# interface Gi1/0/2 Core-SW2(config-if)# switchport mode trunk Core-SW2(config-if)# switchport trunk allowed vlan 10,20,30 Core-SW2(config-if)# exit Core-SW2(config)# interface Gi1/0/24 Core-SW2(config-if)# switchport mode trunk Core-SW2(config-if)# switchport trunk allowed vlan 10,20,30 Core-SW2(config-if)# exit Core-SW2(config)# interface Gi1/0/1 Core-SW2(config-if)# no switchport Core-SW2(config-if)# ip address 10.0.0.6 255.255.255.252 Core-SW2(config-if)# no shutdown Core-SW2(config-if)# exit Core-SW2(config)# end Core-SW2# write memory

Phase 4: Configure the Routers Set up WAN interfaces, OSPF routing, and dual-ISP failover

The routers are where the magic of ISP failover happens. Each router connects to one ISP on its WAN side and to a core switch on its LAN side. You will use OSPF to share routing information between the routers and use floating static routes to make ISP failover automatic.

## Configure Router-R1 (Primary ISP via ISP1-Rogers)

### Step A — Basic setup and WAN interface

Router> enable Router# configure terminal Router(config)# hostname Router-R1 Router-R1(config)# interface Se0/1/0 Router-R1(config-if)# ip address 203.0.113.2 255.255.255.252 Router-R1(config-if)# no shutdown Router-R1(config-if)# exit

### Step B — LAN interface connecting to Core-SW1

Router-R1(config)# interface Gi0/0 Router-R1(config-if)# ip address 10.0.0.1 255.255.255.252 Router-R1(config-if)# no shutdown Router-R1(config-if)# exit

### Step C — Configure OSPF

OSPF (Open Shortest Path First) is a routing protocol that lets routers and switches automatically learn routes from each other. Instead of manually typing every route, OSPF discovers them automatically and updates itself if the network changes.

Router-R1(config)# router ospf 1 Router-R1(config-router)# router-id 1.1.1.1 Router-R1(config-router)# network 10.0.0.0 0.0.0.3 area 0 Router-R1(config-router)# default-information originate Router-R1(config-router)# exit

Command breakdown: 'network 10.0.0.0 0.0.0.3 area 0' tells OSPF to advertise the 10.0.0.0/30 link (connecting to Core-SW1) to all OSPF neighbors. The 0.0.0.3 is a wildcard mask (inverse of the subnet mask). 'default-information originate' tells this router to share its default internet route with all other OSPF devices.

### Step D — Primary default route (ISP1 as main internet path)

This route tells the router to send all internet traffic to ISP1. The administrative distance of 1 makes it the preferred path:

Router-R1(config)# ip route 0.0.0.0 0.0.0.0 203.0.113.1 1

What is a default route? 0.0.0.0 0.0.0.0 means 'any destination not in the routing table.' It is the catch-all route. Any packet with an unknown destination gets sent to the next-hop IP (203.0.113.1 = ISP1). The number at the end (1) is the administrative distance — lower means more preferred.

### Step E — Floating static route (ISP2 as backup — this is the failover!)

This is the key command for dual-ISP failover. It tells the router to use ISP2 ONLY if ISP1 goes down. The higher administrative distance (200) means it is always less preferred than the primary route (1):

Router-R1(config)# ip route 0.0.0.0 0.0.0.0 10.0.0.5 200

How failover works: When ISP1 is up, only the route with AD=1 is active. The AD=200 route is installed but hidden. The moment ISP1's link goes down, the AD=1 route disappears from the routing table. The AD=200 route immediately becomes active and traffic flows through R2 to ISP2 automatically. No human intervention needed.

Critical: The next-hop 10.0.0.5 is the IP address of Core-SW2, not ISP2 directly. R1 reaches ISP2 by sending traffic through the Core switches to R2, which then forwards it out to ISP2. This is why the cross-link between Core-SW1 and Core-SW2 is essential.

### Step F — Save

Router-R1(config)# end Router-R1# write memory

## Configure Router-R2 (Secondary ISP via ISP2-ATT)

R2 is the backup router. It connects to ISP2 and mirrors most of R1's configuration, just with different IP addresses:

Router> enable Router# configure terminal Router(config)# hostname Router-R2 Router-R2(config)# interface Se0/1/0 Router-R2(config-if)# ip address 198.51.100.2 255.255.255.252 Router-R2(config-if)# no shutdown Router-R2(config-if)# exit Router-R2(config)# interface Gi0/0 Router-R2(config-if)# ip address 10.0.0.5 255.255.255.252 Router-R2(config-if)# no shutdown Router-R2(config-if)# exit Router-R2(config)# router ospf 1 Router-R2(config-router)# router-id 2.2.2.2 Router-R2(config-router)# network 10.0.0.4 0.0.0.3 area 0 Router-R2(config-router)# default-information originate Router-R2(config-router)# exit Router-R2(config)# ip route 0.0.0.0 0.0.0.0 198.51.100.1 1 Router-R2(config)# ip route 0.0.0.0 0.0.0.0 10.0.0.1 200 Router-R2(config)# end Router-R2# write memory

R2 floating route: Just like R1 has a floating route through R2 as backup, R2 has a floating route through R1 (10.0.0.1) as its own backup. This gives you full redundancy in both directions.

## Configure OSPF on Core Switches

The core switches must also run OSPF so they can share VLAN subnet routes with the routers:

### Core-SW1 OSPF

Core-SW1# configure terminal Core-SW1(config)# router ospf 1 Core-SW1(config-router)# router-id 3.3.3.3 Core-SW1(config-router)# network 10.0.0.0 0.0.0.3 area 0 Core-SW1(config-router)# network 192.168.10.0 0.0.0.255 area 0 Core-SW1(config-router)# network 192.168.20.0 0.0.0.255 area 0 Core-SW1(config-router)# network 192.168.30.0 0.0.0.255 area 0 Core-SW1(config-router)# exit Core-SW1(config)# end Core-SW1# write memory

### Core-SW2 OSPF

Core-SW2# configure terminal Core-SW2(config)# router ospf 1 Core-SW2(config-router)# router-id 4.4.4.4 Core-SW2(config-router)# network 10.0.0.4 0.0.0.3 area 0 Core-SW2(config-router)# exit Core-SW2(config)# end Core-SW2# write memory

Phase 5: Configure End Devices & DHCP Set PCs to use DHCP and verify they receive addresses

## Configure PCs on Each Floor

Click on each PC > Desktop tab > IP Configuration. Set them to DHCP (not Static). After a few seconds, the PC should receive an IP address from Core-SW1 automatically.

| 1 | Set Floor 1 PCs to DHCP |
| --- | --- |

- Click PC > Desktop > IP Configuration

- Select the DHCP radio button

- Wait 5-10 seconds. The PC should receive 192.168.10.x

- Verify: Default Gateway should show 192.168.10.1

| 2 | Set Floor 2 and Floor 3 PCs to DHCP |
| --- | --- |

Repeat the same process for all PCs on Floor 2 and Floor 3. Floor 2 should receive 192.168.20.x and Floor 3 should receive 192.168.30.x.

If a PC does not get an IP: Check that the port it connects to on the access switch is assigned to the correct VLAN with 'switchport access vlan X'. Also verify the trunk link to Core-SW1 is configured correctly. Use 'show vlan brief' on the access switch to check.

## Configure Printers (Static IPs)

Printers should have static IPs so their address never changes. Assign them addresses from the reserved range you excluded from DHCP:

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Printer-1 (Floor 1) | 192.168.10.5 | 255.255.255.0 | 192.168.10.1 |
| Printer-2 (Floor 3) | 192.168.30.5 | 255.255.255.0 | 192.168.30.1 |
| Printer-3 (Floor 2) | 192.168.20.5 | 255.255.255.0 | 192.168.20.1 |

Phase 6: Test & Verify Everything Use ping, show commands, and simulate ISP failure

## Verification Commands

Run these commands on each device to confirm everything is working. Type them in the CLI tab of each device.

### Check OSPF neighbors are formed

Run this on Router-R1 and Core-SW1. You should see your neighbors listed as FULL state:

Router-R1# show ip ospf neighbor Neighbor ID Pri State Dead Time Address Interface 3.3.3.3 1 FULL/DR 00:00:38 10.0.0.2 GigabitEthernet0/0

Good sign: If you see 'FULL' in the State column, OSPF is working correctly between those two devices. If you see 'INIT' or nothing, check IP addresses and 'no shutdown' on both interfaces.

### Check the routing table

This shows all routes the router knows about. You should see OSPF routes (marked O) for all VLAN subnets:

Router-R1# show ip route Codes: C - connected, S - static, O - OSPF ... Gateway of last resort is 203.0.113.1 to network 0.0.0.0 S* 0.0.0.0/0 [1/0] via 203.0.113.1 C 10.0.0.0/30 is directly connected, GigabitEthernet0/0 O 192.168.10.0/24 [110/2] via 10.0.0.2 O 192.168.20.0/24 [110/2] via 10.0.0.2 O 192.168.30.0/24 [110/2] via 10.0.0.2

### Check VLANs on access switches

ACC-SW1# show vlan brief VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active 10 Floor1-Users active Fa0/2, Fa0/3, Fa0/4

### Ping tests — do these in order

Use the ping command from the Command Prompt on each PC (Desktop > Command Prompt). Work through these tests in order — if an early one fails, fix it before moving on:

| Test | What to ping from Floor 1 PC | What success looks like |
| --- | --- | --- |
| 1. Ping the default gateway | ping 192.168.10.1 | Reply from 192.168.10.1 |
| 2. Ping a Floor 2 PC (inter-VLAN) | ping 192.168.20.x | Reply from 192.168.20.x |
| 3. Ping a Floor 3 PC | ping 192.168.30.x | Reply from 192.168.30.x |
| 4. Ping Router-R1 | ping 10.0.0.1 | Reply from 10.0.0.1 |
| 5. Ping the ISP1 (internet test) | ping 203.0.113.1 | Reply from 203.0.113.1 |

## Test ISP Failover — The Main Event

This is the most important test. You will simulate ISP1 going down and verify that traffic automatically reroutes through ISP2.

| 1 | Check current active path |
| --- | --- |

Router-R1# show ip route 0.0.0.0 S* 0.0.0.0/0 [1/0] via 203.0.113.1

This confirms traffic is going out through ISP1 (203.0.113.1) with AD=1.

| 2 | Simulate ISP1 failure — shut down the WAN interface |
| --- | --- |

Router-R1# configure terminal Router-R1(config)# interface Se0/1/0 Router-R1(config-if)# shutdown

| 3 | Check the routing table again immediately |
| --- | --- |

Router-R1# show ip route 0.0.0.0 S* 0.0.0.0/0 [200/0] via 10.0.0.5

Failover confirmed! The route now shows AD=200 via 10.0.0.5 (Core-SW2/R2 path). Traffic is automatically flowing through ISP2 without any manual intervention. This is exactly how enterprise dual-ISP failover works in real life.

| 4 | Re-enable ISP1 and watch it restore |
| --- | --- |

Router-R1(config)# interface Se0/1/0 Router-R1(config-if)# no shutdown Router-R1(config-if)# end Router-R1# show ip route 0.0.0.0 S* 0.0.0.0/0 [1/0] via 203.0.113.1

The primary ISP1 route comes back automatically. The network self-heals without any manual commands.

Phase 7: Final Project Checklist Verify every component is working before calling it done

## Topology Checklist

| ☐ | 2 Cloud devices added and named ISP1-Rogers and ISP2-ATT |
| --- | --- |
| ☐ | 2 ISR4331 routers added with HWIC-2T serial module installed |
| ☐ | 2 x 3650-24PS core switches added |
| ☐ | 3 x 2960 access switches added, one per floor |
| ☐ | End devices (PCs, printers, APs) added to all three floors |
| ☐ | All cables connected and port lights are showing green |

## Configuration Checklist

| ☐ | Access switches: VLANs 10, 20, 30 created and named |
| --- | --- |
| ☐ | Access switches: Access ports assigned to correct VLANs |
| ☐ | Access switches: Uplink ports configured as trunk ports |
| ☐ | Core-SW1: ip routing enabled |
| ☐ | Core-SW1: SVI created for VLAN 10, 20, and 30 with correct IPs |
| ☐ | Core-SW1: DHCP pools configured for all three floors |
| ☐ | Core-SW1: Excluded addresses configured (.1 to .10 in each subnet) |
| ☐ | Core-SW1 and Core-SW2: OSPF configured with correct network statements |
| ☐ | Core-SW1 and Core-SW2: Trunk link between them configured |
| ☐ | Router-R1: WAN (Se0/1/0) and LAN (Gi0/0) interfaces configured with IPs |
| ☐ | Router-R1: Primary default route via ISP1 (AD=1) |
| ☐ | Router-R1: Floating static route via Core-SW2 (AD=200) |
| ☐ | Router-R1: OSPF configured with router-id and network statements |
| ☐ | Router-R2: Same as R1 but with ISP2 IPs |
| ☐ | All devices: write memory run after configuration |

## Testing Checklist

| ☐ | All PCs received IP addresses via DHCP in the correct subnet |
| --- | --- |
| ☐ | Printers assigned static IPs within excluded range |
| ☐ | Floor 1 PC can ping its gateway (192.168.10.1) |
| ☐ | Floor 1 PC can ping Floor 2 PC (inter-VLAN routing works) |
| ☐ | Floor 1 PC can ping Floor 3 PC |
| ☐ | Any PC can ping Router-R1 (10.0.0.1) |
| ☐ | Any PC can ping ISP1 cloud IP (203.0.113.1) |
| ☐ | OSPF neighbors show as FULL state on all devices |
| ☐ | Shutting down R1 Se0/1/0 causes failover to AD=200 route |
| ☐ | Re-enabling R1 Se0/1/0 restores the primary AD=1 route automatically |

## Troubleshooting Quick Reference

| Problem | Likely Cause | Fix |
| --- | --- | --- |
| PC not getting DHCP IP | VLAN not assigned to port or trunk missing | Run 'show vlan brief' on access switch, check trunk with 'show interfaces trunk' |
| Inter-VLAN ping failing | 'ip routing' not enabled on core switch | Run 'ip routing' on core switch CLI |
| OSPF neighbors not forming | IP mismatch or interface shutdown | Check IPs match /30 subnet, run 'no shutdown' on both sides |
| Failover not working | Floating route AD wrong or next-hop unreachable | Check AD=200 on backup route, verify Core-SW2 is reachable |
| Can't ping ISP | Default route not configured or WAN IP wrong | Run 'show ip route' and verify 0.0.0.0/0 is present |

Congratulations! You have built a production-grade multi-floor enterprise network with dual ISP failover. The concepts you practiced — VLANs, trunking, OSPF, SVI routing, DHCP, and floating static routes — are used every day by network engineers worldwide.